Product name Safari 20SG

EPA Reg # 86203-11-59639

Manufacture Valent

Active Ingredient Dinotefuran

Signal Word Caution

Emergency phone #  800-892-0099

Precautionary statements:

Human and Animal

Harmful if swallowed or absorbed through skin. Avoid contact with eyes, skin or clothing. Wash thoroughly with soap and water after handling and before eating, drinking, chewing gum, using tobacco or using the toilet. Causes moderate eye irritation. Remove and wash contaminated clothing before reuse.

Environmental

This pesticide is toxic to aquatic invertebrates. Do not apply directly to water, or to areas where surface water is present or to intertidal areas below the mean high water mark. Do not apply when weather conditions favor drift from treated areas. Drift and runoff from treated areas may be hazardous to aquatic organisms in water adjacent to treated areas. Do not dispose of equipment washwaters or

rinsate into a natural drain or water body. This product is toxic to honey bees. The persistence

of residues and potential residual toxicity of dinotefuran in nectar and pollen suggests the possibility of chronic toxic risk to honey bee larvae and the eventual instability of the hive.

• This product is toxic to bees exposed to residues

for more than 38 hours following treatment.

• Do not apply this product to blooming, pollen-shedding or nectar-producing parts of plants if bees

may forage on the plants during this time period, unless the application is made in response to a

public health emergency declared by appropriate state or federal authorities. Dinotefuran and its degradate, MNG, have the properties and characteristics associated with chemicals detected in groundwater. The high water solubility of dinotefuran, and its degradate, MNG, coupled with

its very high mobility, and resistance to biodegradation indicates that this compound has a strong potential to leach to the subsurface under certain conditions as a result of label use. Use of this chemical

in areas where soils are permeable, particularly where the water table is shallow, may result in

groundwater contamination.

First Aid

If on skin or clothing:

- Take off contaminated clothing.

- Rinse skin immediately with plenty of water for 15-20 minutes.

- Call a poison control center or doctor for further treatment advice.

If swallowed:

- Call poison control center or doctor immediately for treatment advice.

- Do not induce vomiting unless told to do so by the poison control center or doctor.

- Have person sip a glass of water if able to swallow.

- Do not give anything by mouth to an unconscious person.

If in eyes:

- Hold eye open and rinse slowly and gently with water for 15-20 minutes.

- Remove contact lenses, if present, after the first 5 minutes, then continue rinsing eye.

- Call a poison control center or doctor for further treatment advice.

If inhaled:

- Move person to fresh air.

- If person is not breathing, call 911 or an ambulance, then give artificial respiration, preferably mouth-to-mouth, if possible.

- Call poison control center or doctor for further treatment advice.

Re-entry information

Do not allow others to enter treated areas until sprays have dried.